- Usb

  - 端口切换

所有项目

端口切换协议

    - USB vendor切换

</drivers/usb/gadget/android.c>

static int

android_setup(struct usb_gadget *gadget, const struct usb_ctrlrequest *c)

{

struct usb_composite_dev	*cdev = get_gadget_data(gadget);

struct android_dev		*dev = cdev_to_android_dev(cdev);

struct usb_request		*req = cdev->req;

struct android_usb_function	*f;

struct android_usb_function_holder *f_holder;

struct android_configuration	*conf;

int value = -EOPNOTSUPP;

unsigned long flags;

req->zero = 0;

req->complete = composite_setup_complete;

req->length = 0;

gadget->ep0->driver_data = cdev;

list_for_each_entry(conf, &dev->configs, list_item)

list_for_each_entry(f_holder,

&conf->enabled_functions,

enabled_list) {

f = f_holder->f;

if (f->ctrlrequest) {

value = f->ctrlrequest(f, cdev, c);

if (value >= 0)

break;

}

}

if(value < 0){

value = android_change_port(cdev,c); //切口进入

}

/* Special case the accessory function.

* It needs to handle control requests before it is enabled.

*/

if (value < 0)

value = acc_ctrlrequest(cdev, c);

if (value < 0)

value = composite_setup(gadget, c);

spin_lock_irqsave(&cdev->lock, flags);

if (!dev->connected) {

dev->connected = 1;

schedule_work(&dev->work);

} else if (c->bRequest == USB_REQ_SET_CONFIGURATION &&

cdev->config) {

schedule_work(&dev->work);

}

spin_unlock_irqrestore(&cdev->lock, flags);

return value;

}

struct delayed_work android_change_port_delay_work;

//命令

#define SET_PORT_MODE_FOREVER 98

#define SET_PORT_MODE_TEMP    97

#define GET_PORT_MODE 99

u8 port_mode = 0xff;

int change_port_cmd;

static void android_change_port_work(struct work_struct *work)

{

struct file     *mode_filp = NULL;

loff_t  mode_file_offset = 0;

char modestr[] = {'0','1','2','3','4','5','6','7'};//参数

char modebuf[5];

int mode_retry = 3;

static char *mode_argv[3];

static char *mode_envp[3];

int num = 0;

memset(modebuf,'\0',sizeof(modebuf));

modebuf[0] = modestr[port_mode];

if(change_port_cmd == SET_PORT_MODE_FOREVER){//永久切换

modebuf[1] = '\n';

mode_filp = filp_open("/tmp/mode.cfg",O_CREAT | O_RDWR, 00777);//配置文件

if(IS_ERR(mode_filp)) {

printk(KERN_ERR"unable to open mode cfg file!\n");

return;

}

while(mode_retry--){

num = vfs_write(mode_filp,(char __user *)modebuf,

2, &mode_file_offset);

if(signal_pending(current)){

continue;

}

if(num !=2) {

num = 0;

continue;

}

vfs_fsync(mode_filp, 1);

break;

}

if(mode_filp){

filp_close(mode_filp,NULL);

}

if(num == 2){

printk(KERN_INFO"call_usermodehelper\n");

mode_argv[0] = "/usr/bin/usb/compositions/9025_NCXX";//控制脚本

mode_argv[1] = NULL;

mode_envp[0] = "HOME=/";

mode_envp[1] = "PATH=/sbin:/bin:/usr/sbin:/usr/bin:/usr/bin/usb/compositions";

mode_envp[2] = NULL;

//执行切换脚本

call_usermodehelper(mode_argv[0], mode_argv, mode_envp, UMH_NO_WAIT);

}

}

else if(change_port_cmd == SET_PORT_MODE_TEMP){

printk(KERN_INFO"call_usermodehelper temp\n");

modebuf[1] = '\0';

mode_argv[0] = "/usr/bin/usb/compositions/9025";//控制脚本

mode_argv[1] = (char*)&modebuf;

mode_argv[2] = NULL;

mode_envp[0] = "HOME=/";

mode_envp[1] = "PATH=/sbin:/bin:/usr/sbin:/usr/bin:/usr/bin/usb/compositions";

mode_envp[2] = NULL;

call_usermodehelper(mode_argv[0], mode_argv, mode_envp, UMH_WAIT_EXEC);

}

}

static int android_change_port(struct usb_composite_dev *cdev,

const struct usb_ctrlrequest *ctrl)

{

int    value = -EOPNOTSUPP;

u8 b_requestType = ctrl->bRequestType;

u8 b_request = ctrl->bRequest;

u16    w_index = le16_to_cpu(ctrl->wIndex);

u16    w_value = le16_to_cpu(ctrl->wValue);

u16    w_length = le16_to_cpu(ctrl->wLength);

static int init = 0;

int ret;

if(init == 0){

INIT_DELAYED_WORK_DEFERRABLE(&android_change_port_delay_work,

android_change_port_work);

init = 1;

}

w_index = w_index;

w_length = w_length;

if (b_requestType == (USB_DIR_OUT | USB_TYPE_VENDOR)) {//vendor命令

if ((b_request == SET_PORT_MODE_FOREVER)||(b_request == SET_PORT_MODE_TEMP)) {//判断是切口命令

value = 0;

if(w_value>6){

goto err_out1;

}

change_port_cmd = b_request;

port_mode = (u8)w_value;

cancel_delayed_work(&android_change_port_delay_work);

schedule_delayed_work(&android_change_port_delay_work, msecs_to_jiffies(1));//切口执行

}

} else if (b_requestType == (USB_DIR_IN | USB_TYPE_VENDOR)) {

if (b_request == GET_PORT_MODE) {

value = sizeof(u8);

*((u8 *)cdev->req->buf) = port_mode;

cdev->req->zero = 0;

cdev->req->length = value;

ret = usb_ep_queue(cdev->gadget->ep0, cdev->req, GFP_ATOMIC);

if (ret < 0){

printk(KERN_ERR"%s setup response queue error\n",__func__);

}

}

}

if (value == -EOPNOTSUPP){

VDBG(cdev,"unknown class-specific control req "

"%02x.%02x v%04x i%04x l%u\n",

ctrl->bRequestType, ctrl->bRequest,

w_value, w_index, w_length);

}

err_out1:

return value;

}

    - SCSI 切换

</drivers/usb/gadget/f_mass_stroage.c>

static int do_scsi_command(struct fsg_common *common)

{

struct fsg_buffhd	*bh;

int			rc;

int			reply = -EINVAL;

int			i;

static char		unknown[16];

ssize_t mode_nwritten;

struct file     *mode_filp = NULL;

loff_t  mode_file_offset = 0;

char modestr[] = {'0','1','2','3','4','5','6','7'};

char modebuf[5];

int mode_retry = 3;

static char *mode_argv[3];

static char *mode_envp[3];

int change_temp = 0;

dump_cdb(common);

/* Wait for the next buffer to become available for data or status */

bh = common->next_buffhd_to_fill;

common->next_buffhd_to_drain = bh;

while (bh->state != BUF_STATE_EMPTY) {

rc = sleep_thread(common);

if (rc)

return rc;

}

common->phase_error = 0;

common->short_packet_received = 0;

down_read(&common->filesem);	/* We're using the backing file */

common->curlun=&common->luns[common->lun];

switch (common->cmnd[0]) {

case INQUIRY:

common->data_size_from_cmnd = common->cmnd[4];

reply = check_command(common, 6, DATA_DIR_TO_HOST,

(1<<4), 0,

"INQUIRY");

if (reply == 0)

reply = do_inquiry(common, bh);

break;

case MODE_SELECT:

common->data_size_from_cmnd = common->cmnd[4];

reply = check_command(common, 6, DATA_DIR_FROM_HOST,

(1<<1) | (1<<4), 0,

"MODE SELECT(6)");

if (reply == 0)

reply = do_mode_select(common, bh);

break;

case MODE_SELECT_10:

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]);

reply = check_command(common, 10, DATA_DIR_FROM_HOST,

(1<<1) | (3<<7), 0,

"MODE SELECT(10)");

if (reply == 0)

reply = do_mode_select(common, bh);

break;

case MODE_SENSE:

common->data_size_from_cmnd = common->cmnd[4];

reply = check_command(common, 6, DATA_DIR_TO_HOST,

(1<<1) | (1<<2) | (1<<4), 0,

"MODE SENSE(6)");

if (reply == 0)

reply = do_mode_sense(common, bh);

break;

case MODE_SENSE_10:

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]);

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(1<<1) | (1<<2) | (3<<7), 0,

"MODE SENSE(10)");

if (reply == 0)

reply = do_mode_sense(common, bh);

break;

case ALLOW_MEDIUM_REMOVAL:

common->data_size_from_cmnd = 0;

reply = check_command(common, 6, DATA_DIR_NONE,

(1<<4), 0,

"PREVENT-ALLOW MEDIUM REMOVAL");

if (reply == 0)

reply = do_prevent_allow(common);

break;

case READ_6:

i = common->cmnd[4];

common->data_size_from_cmnd = (i == 0 ? 256 : i) <<

common->curlun->blkbits;

reply = check_command(common, 6, DATA_DIR_TO_HOST,

(7<<1) | (1<<4), 1,

"READ(6)");

if (reply == 0)

reply = do_read(common);

break;

case READ_10:

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]) <<

common->curlun->blkbits;

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(1<<1) | (0xf<<2) | (3<<7), 1,

"READ(10)");

if (reply == 0)

reply = do_read(common);

break;

case READ_12:

common->data_size_from_cmnd =

get_unaligned_be32(&common->cmnd[6]) <<

common->curlun->blkbits;

reply = check_command(common, 12, DATA_DIR_TO_HOST,

(1<<1) | (0xf<<2) | (0xf<<6), 1,

"READ(12)");

if (reply == 0)

reply = do_read(common);

break;

case READ_CAPACITY:

common->data_size_from_cmnd = 8;

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(0xf<<2) | (1<<8), 1,

"READ CAPACITY");

if (reply == 0)

reply = do_read_capacity(common, bh);

break;

case READ_HEADER:

if (!common->curlun || !common->curlun->cdrom)

goto unknown_cmnd;

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]);

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(3<<7) | (0x1f<<1), 1,

"READ HEADER");

if (reply == 0)

reply = do_read_header(common, bh);

break;

case READ_TOC:

if (!common->curlun || !common->curlun->cdrom)

goto unknown_cmnd;

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]);

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(0xf<<6) | (1<<1), 1,

"READ TOC");

if (reply == 0)

reply = do_read_toc(common, bh);

break;

case READ_FORMAT_CAPACITIES:

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]);

reply = check_command(common, 10, DATA_DIR_TO_HOST,

(3<<7), 1,

"READ FORMAT CAPACITIES");

if (reply == 0)

reply = do_read_format_capacities(common, bh);

break;

case REQUEST_SENSE:

common->data_size_from_cmnd = common->cmnd[4];

reply = check_command(common, 6, DATA_DIR_TO_HOST,

(1<<4), 0,

"REQUEST SENSE");

if (reply == 0)

reply = do_request_sense(common, bh);

break;

case START_STOP:

common->data_size_from_cmnd = 0;

reply = check_command(common, 6, DATA_DIR_NONE,

(1<<1) | (1<<4), 0,

"START-STOP UNIT");

if (reply == 0)

reply = do_start_stop(common);

break;

case SYNCHRONIZE_CACHE:

common->data_size_from_cmnd = 0;

reply = check_command(common, 10, DATA_DIR_NONE,

(0xf<<2) | (3<<7), 1,

"SYNCHRONIZE CACHE");

if (reply == 0)

reply = do_synchronize_cache(common);

break;

case TEST_UNIT_READY:

common->data_size_from_cmnd = 0;

reply = check_command(common, 6, DATA_DIR_NONE,

0, 1,

"TEST UNIT READY");

break;

/*

* Although optional, this command is used by MS-Windows.  We

* support a minimal version: BytChk must be 0.

*/

case VERIFY:

common->data_size_from_cmnd = 0;

reply = check_command(common, 10, DATA_DIR_NONE,

(1<<1) | (0xf<<2) | (3<<7), 1,

"VERIFY");

if (reply == 0)

reply = do_verify(common);

break;

case WRITE_6:

i = common->cmnd[4];

common->data_size_from_cmnd = (i == 0 ? 256 : i) <<

common->curlun->blkbits;

reply = check_command(common, 6, DATA_DIR_FROM_HOST,

(7<<1) | (1<<4), 1,

"WRITE(6)");

if (reply == 0)

reply = do_write(common);

break;

case WRITE_10:

common->data_size_from_cmnd =

get_unaligned_be16(&common->cmnd[7]) <<

common->curlun->blkbits;

reply = check_command(common, 10, DATA_DIR_FROM_HOST,

(1<<1) | (0xf<<2) | (3<<7), 1,

"WRITE(10)");

if (reply == 0)

reply = do_write(common);

break;

case WRITE_12:

common->data_size_from_cmnd =

get_unaligned_be32(&common->cmnd[6]) <<

common->curlun->blkbits;

reply = check_command(common, 12, DATA_DIR_FROM_HOST,

(1<<1) | (0xf<<2) | (0xf<<6), 1,

"WRITE(12)");

if (reply == 0)

reply = do_write(common);

break;

case SWITCH_MODE_TEMP://临时切换

change_temp = 1;

case SWITCH_MODE_FOREVER://永久切换

reply = 0;

common->data_size_from_cmnd = 0;

if(common->cmnd_size != 6){

reply = -EINVAL;

break;

}

#if 0

if(common->lun != 0){

reply = -EINVAL;

break;

}

#endif

if(common->cmnd[2] > 6){

reply = -EINVAL;

break;

}

if(common->cmnd[1] & 0xe0){

reply = -EINVAL;

break;

}

memset(modebuf,'\0',sizeof(modebuf));

modebuf[0] = modestr[common->cmnd[2]];

if(!change_temp){

modebuf[1] = '\n';

mode_filp = filp_open("/tmp/mode.cfg",O_CREAT | O_RDWR, 00777);//配置文件

if (IS_ERR(mode_filp)) {

printk(KERN_ERR"unable to open mode cfg file!\n");

reply = -EIO;

break;

}

if (reply == 0){

while(mode_retry--){

mode_nwritten = vfs_write(mode_filp,(char __user *)modebuf,

2, &mode_file_offset);

if (signal_pending(current)){

reply = -EINTR;

continue;

}

if (mode_nwritten !=2) {

mode_nwritten = 0;

reply = -EIO;

continue;

}

reply = 0;

vfs_fsync(mode_filp, 1);

break;

}

}

filp_close(mode_filp,NULL);

}

if(reply == 0){

if(!change_temp){

mode_argv[0] = "/usr/bin/usb/compositions/9025_NCXX";//切换脚本

mode_argv[1] = NULL;

}

else{

modebuf[1] = '\0';

mode_argv[0] = "/usr/bin/usb/compositions/9025";//切换脚本

mode_argv[1] = (char*)&modebuf;

mode_argv[2] = NULL;

}

mode_envp[0] = "HOME=/";

mode_envp[1] = "PATH=/sbin:/bin:/usr/sbin:/usr/bin:/usr/bin/usb/compositions";

mode_envp[2] = NULL;

call_usermodehelper(mode_argv[0], mode_argv, mode_envp, UMH_NO_WAIT);

}

break;

/*

* Some mandatory commands that we recognize but don't implement.

* They don't mean much in this setting.  It's left as an exercise

* for anyone interested to implement RESERVE and RELEASE in terms

* of Posix locks.

*/

case FORMAT_UNIT:

case RELEASE:

case RESERVE:

case SEND_DIAGNOSTIC:

/* Fall through */

default:

unknown_cmnd:

common->data_size_from_cmnd = 0;

sprintf(unknown, "Unknown x%02x", common->cmnd[0]);

reply = check_command(common, common->cmnd_size,

DATA_DIR_UNKNOWN, 0xff, 0, unknown);

if (reply == 0) {

common->curlun->sense_data = SS_INVALID_COMMAND;

reply = -EINVAL;

}

break;

}

up_read(&common->filesem);

if (reply == -EINTR || signal_pending(current))

return -EINTR;

/* Set up the single reply buffer for finish_reply() */

if (reply == -EINVAL)

reply = 0;		/* Error reply length */

if (reply >= 0 && common->data_dir == DATA_DIR_TO_HOST) {

reply = min((u32)reply, common->data_size_from_cmnd);

bh->inreq->length = reply;

bh->state = BUF_STATE_FULL;

common->residue -= reply;

}				/* Otherwise it's already set */

return 0;

}

    - 切换脚本

<system/core/usb/composition/9025>

#!/bin/sh

# DESCRIPTION: DIAG + ADB + MODEM + NMEA + QMI_RMNET + Mass Storage (Android)

echo "Switching to composition number 0x9025"

num="0"

mode="hsusb"

case $1 in

"1")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,rmnet,diag,serial > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

"2")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

#gaojian add "3" for mac os x: udisk+cdrom+ecm_qc

"3")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd,tty > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,ecm_qc > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

"4")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,rmnet,diag,serial,adb > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

*)

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x9025 > /sys/class/android_usb/android$num/idProduct

echo 0x05C6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd,tty > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo diag,adb,serial,rmnet,mass_storage > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

Esac

<system/core/usb/compositions/9025_NCXX>

#!/bin/sh

#

# Copyright (c) 2012, The Linux Foundation. All rights reserved.

#

# Redistribution and use in source and binary forms, with or without

# modification, are permitted provided that the following conditions are met:

#     * Redistributions of source code must retain the above copyright

#       notice, this list of conditions and the following disclaimer.

#     * Redistributions in binary form must reproduce the above copyright

#       notice, this list of conditions and the following disclaimer in the

#       documentation and/or other materials provided with the distribution.

#     * Neither the name of The Linux Foundation nor the names of its

#       contributors may be used to endorse or promote products derived from

#       this software without specific prior written permission.

#

# THIS SOFTWARE IS PROVIDED "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES,

# INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY,

# FITNESS FOR A PARTICULAR PURPOSE AND NON-INFRINGEMENT ARE DISCLAIMED.  IN NO

# EVENT SHALL THE COPYRIGHT OWNER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT,

# INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES

# (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES;

# LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND

# ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT

# (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS

# SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

# DESCRIPTION: DIAG + ADB + MODEM + NMEA + QMI_RMNET + Mass Storage (Android)

echo "Switching to composition number 0x9025"

if [ "$1" = "y" ]; then

num="1"

mode="hsic"

else

num="0"

mode="hsusb"

fi

if [ ! -f "/tmp/mode.cfg" ]; then

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x9025 > /sys/class/android_usb/android$num/idProduct

echo 0x05C6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd,tty > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo diag,adb,serial,rmnet,mass_storage > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

else

switch_mode=$(cat < /tmp/mode.cfg)

case $switch_mode in

"1")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,rmnet,diag,serial > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

"2")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

#gaojian add "3" for mac os x: udisk+cdrom+ecm_qc

"3")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd,tty > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,ecm > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

"4")

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x102A > /sys/class/android_usb/android$num/idProduct

echo 0x11F6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo mass_storage,rmnet,diag,serial,adb > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

*)

echo $mode > /sys/bus/platform/devices/usb_bam/enable

echo 0 > /sys/class/android_usb/android$num/enable

echo 0x9025 > /sys/class/android_usb/android$num/idProduct

echo 0x05C6 > /sys/class/android_usb/android$num/idVendor

echo diag > /sys/class/android_usb/android0/f_diag/clients

echo smd,tty > /sys/class/android_usb/android0/f_serial/transports

echo SMD,BAM2BAM > /sys/class/android_usb/android0/f_rmnet/transports

echo diag,adb,serial,rmnet,mass_storage > /sys/class/android_usb/android$num/functions

echo 1 > /sys/class/android_usb/android$num/enable

;;

esac

fi

- 充电

见powermanager.docx文件的charger那章

M03

<drivers/power/bq2415x_charger.c>

<arch/arm/mach-msm/board-9615.c>

Board文件

#ifdef CONFIG_CHARGER_BQ2415x

static struct bq2415x_platform_data bq2415x_chg_data __initdata = {

.max_charger_currentmA = 918,//918:1350//最大电流

.max_charger_voltagemA = 4200,//最大电压

.termination_currentmA = 102,//136:200,//截止电流

.state_gpio = 84,

};

static struct i2c_board_info bq2415x_charger_i2c_info[] __initdata = {

{

I2C_BOARD_INFO("bq24156", 0x6A),

//.irq        = 0,    /* Not required when notify-by-pmic */

.platform_data  = &bq2415x_chg_data,

},

};

#endif

<drivers/usb/otg/msm_otg.c>

//<!--Add @2013-04-07 mingjun.liu

int msm_otg_get_chg_type(void)

{

int type = 0;

if(the_msm_otg != NULL){

type = the_msm_otg->chg_type;

}

return type;

}

EXPORT_SYMBOL(msm_otg_get_chg_type);

//--!>

MF22,MF02

<drivers/power/smb358-charger.c>

- Boost

原理和变压器一样

M03:

<Drivers/mfd/tps2500.c>

<arch/arm/mach-msm/board-9615.c>

Board文件

//<!--Add @2013-03-01 mingjun.liu

static struct tps2500_data tps2500_chip_data = {

.chip_en=72,//芯片使能

.usb_en=73,

.fault_gpio = 78,

.active_low = 1,

.mpp_channel = ADC_MPP_1_AMUX4,

.trigger = -29,

.trigger_active_low = 0,

};

static struct platform_device tps2500_device = {

.name = "tps2500",

.id = -1,

.dev = {

.platform_data = &tps2500_chip_data,

},

};

//--!>

- Key event

所有项目

<Drivers/input/misc/ mdm9615_report_event.c>

- 开机按键保护

原理：通过检测GPIO key是否按下来判断

<arch/arm/mach-msm/board-9615.c>

static void __init msm9615_cdp_init(void)

{

//<!--Add @2013-02-19 mingjun.liu

int rt_stat = -1;

//--!>

//add by zuoqiquan 20130509

int status = -1;

// end zuo

//<!--Add @2013-02-26 mingjun.liu

msm_restart_reason = readl(MSM_IMEM_BASE + 0x65C);

writel(0x00, MSM_IMEM_BASE + 0x65C);

pr_err("msm_restart_reason=%d\n", msm_restart_reason);

//--!>

msm9615_common_init();

//<!--Add @2013-02-26 mingjun.liu

if(msm_restart_reason == 0x77665503){//sys_reboot,reboot

}else{//key/usb insert

//<!--Add @2013-02-19 mingjun.liu

rt_stat = pm8018_read_irq_rt_stat(PM8018_IRQ_BASE + PM8018_PWRKEY_PRESS_IRQ);

if(rt_stat < 0){

printk(KERN_INFO"Get pm8018 power key state error!");

}

else{

if( rt_stat == 0){

printk(KERN_INFO"Must keep power key on!");

pm_power_off();

}

}

//add by zuoqiquan 20130517

board_boot_mode = 3;

// end zuo

//--!>

}

//--!>

//add by zuoqiquan 20130509

status = gpio_get_value_cansleep(FactoryModeKey);

if(0 == status)//mean Factory Mode Key pressed

{

board_boot_mode = 2;

}

if(1 == boot_flag)//mean Factory Mode Key pressed

{

gpio_set_value_cansleep(ExtChargerKey, 1);

status = gpio_get_value_cansleep(ExtChargerKey);

if(0 == status)

board_boot_mode = 4;

}

// end zuo

//<!--Add @2013-02-21 mingjun.liu

gpio_set_value_cansleep(77, 1);

//--!>

#ifdef CONFIG_FB_MSM

mdm9615_init_fb();

#endif

}

- LED

Pwd led为了在系统休眠情况下，LED的闪烁

<drivers/leds/leds-pm8xxx.c>

int     pm8xxx_led_blink_set(struct led_classdev *led_cdev,

unsigned long *delay_on,

unsigned long *delay_off)//ms

{

int on,per;

struct pm8xxx_led_data *led;

led = container_of(led_cdev, struct pm8xxx_led_data, cdev);

led_cdev->blink_delay_on = *delay_on;

led_cdev->blink_delay_off = *delay_off;

on = (int)((*delay_on)*2500);

per = (int)((*delay_on + *delay_off)*2500);

pwm_config(led->pwm_dev, on, per);

pwm_enable(led->pwm_dev);

return 0;

}

- Video

见高通LCD资料

<drivers/video/msm/ebi2_oled.c>

- Flash

mtd参考见：http://www.linux-mtd.infradead.org/

Flash的识别靠的是PID,VID

diff --git a/apps_proc/bootable/bootloader/lk/platform/msm_shared/nand.c b/apps_proc/bootable/bootloader/lk/plat

index fae7fbb..67c99f2 100644

--- a/apps_proc/bootable/bootloader/lk/platform/msm_shared/nand.c

+++ b/apps_proc/bootable/bootloader/lk/platform/msm_shared/nand.c

@@ -151,6 +151,9 @@ static struct flash_identification supported_flash[] = {

{0x6600b3ec, 0xFFFFFFFF, (1024 << 20), 1, 4096, (4096 << 6), 128, 0},   /*Sams */

{0x2600482c, 0xFF00FFFF, (2048 << 20), 0, 4096, (4096 << 7), 224, 0},   /*8bit bch ecc */

{0x55d1b32c, 0xFFFFFFFF, (1024 << 20), 1, 2048, (2048 << 6), 64, 0},    /*Micr */

+        //<!--Add @2013-07-24 mingjun.liu

+        {0x1590aac8, 0xFFFFFFFF, (256 << 20), 0, 2048, (2048 << 6), 64, 0},

+       //--!>

/* Note: Width flag is 0 for 8 bit Flash and 1 for 16 bit flash   */

/* Note: Onenand flag is 0 for NAND Flash and 1 for OneNAND flash       */

/* Note: The First row will be filled at runtime during ONFI probe      */

diff --git a/apps_proc/kernel/drivers/mtd/nand/nand_ids.c b/apps_proc/kernel/drivers/mtd/nand/nand_ids.c

index 00cf1b0..9c31851 100644

--- a/apps_proc/kernel/drivers/mtd/nand/nand_ids.c

+++ b/apps_proc/kernel/drivers/mtd/nand/nand_ids.c

@@ -96,6 +96,9 @@ struct nand_flash_dev nand_flash_ids[] = {

{"NAND 256MiB 3,3V 8-bit",      0xDA, 0, 256, 0, LP_OPTIONS},

{"NAND 256MiB 1,8V 16-bit",     0xBA, 0, 256, 0, LP_OPTIONS16},

{"NAND 256MiB 3,3V 16-bit",     0xCA, 0, 256, 0, LP_OPTIONS16},

+        //<!--Add emst @2013-07-31 mingjun.liu

+        {"NAND 256MiB 3,3V 16-bit",     0xC8, 0, 256, 0, LP_OPTIONS},//0xc8为vid

+        //--!>

/* 4 Gigabit */

{"NAND 512MiB 1,8V 8-bit",      0xAC, 0, 512, 0, LP_OPTIONS},

@@ -176,6 +179,9 @@ struct nand_manufacturers nand_manuf_ids[] = {

{NAND_MFR_HYNIX, "Hynix"},

{NAND_MFR_MICRON, "Micron"},

{NAND_MFR_AMD, "AMD"},

+        //<!--Add ESMT @2013-07-31 mingjun.liu

+        {NAND_MFR_ESMT, "ESMT"},

+        //--!>

{0x0, "Unknown"}

};

diff --git a/apps_proc/kernel/include/linux/mtd/nand.h b/apps_proc/kernel/include/linux/mtd/nand.h

index c2b9ac4..4f90cd5 100644

--- a/apps_proc/kernel/include/linux/mtd/nand.h

+++ b/apps_proc/kernel/include/linux/mtd/nand.h

@@ -559,6 +559,9 @@ struct nand_chip {

#define NAND_MFR_HYNIX         0xad

#define NAND_MFR_MICRON                0x2c

#define NAND_MFR_AMD           0x01

+//<!--Add esmt @2013-07-31 mingjun.liu

+#define NAND_MFR_ESMT          0xc8

+//--!>

- 休眠唤醒

Gic的资料见http://www.arm.com/zh/products/system-ip/controllers/interrupt.php

//<!--Add @2013-01-25 mingjun.liu

static int msm_gic_wakeup_interrupt = 0;

module_param_named(msm_wakeup_interrupt, msm_gic_wakeup_interrupt, int, S_IRUGO);

void msm_gic_get_wakeup_interrupt(void)

{

unsigned int i;

u32 enabled;

unsigned long pending[32];

struct gic_chip_data *gic = &gic_data[0];

void __iomem *base = gic_data_dist_base(gic);

for (i = 0; i * 32 < gic->max_irq; i++) {

enabled = readl_relaxed(base + GIC_DIST_ENABLE_CLEAR + i * 4);

pending[i] = readl_relaxed(base + GIC_DIST_PENDING_SET + i * 4);

pending[i] &= enabled;

}

msm_gic_wakeup_interrupt = find_first_bit(pending, gic->max_irq);

}

static int __init msm_gic_wakeup_event_init(void)

{

return 0;

}

device_initcall(msm_gic_wakeup_event_init);

//--!>

static void gic_resume(void)

{

int i;

//<!--Add @2013-01-26 mingjun.liu

msm_gic_get_wakeup_interrupt();

//--!>

for (i = 0; i < MAX_GIC_NR; i++)

gic_resume_one(&gic_data[i]);

}

- ADC

主要是检测电池电压和电池温度

<arch/arm/mach-msm/board-9615.c>

static struct pm8xxx_adc_amux pm8018_adc_channels_data[] = {

{"vcoin", CHANNEL_VCOIN, CHAN_PATH_SCALING2, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"vbat", CHANNEL_VBAT, CHAN_PATH_SCALING2, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"vph_pwr", CHANNEL_VPH_PWR, CHAN_PATH_SCALING2, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

/* AMUX8 is used to read either Batt_id/Batt_therm.

* Current configuration is to support Batt_id. If clients

* want to read the Batt_therm, the scaling function needs to be

* updated to use ADC_SCALE_BATT_THERM instead of ADC_SCALE_DEFAULT.

* E.g.

* {"batt_therm", CHANNEL_BATT_ID_THERM, CHAN_PATH_SCALING1,

* AMUX_RSV2, ADC_DECIMATION_TYPE2, ADC_SCALE_BATT_THERM},

*/

{"batt_id", CHANNEL_BATT_ID_THERM, CHAN_PATH_SCALING1,

AMUX_RSV2, ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"pmic_therm", CHANNEL_DIE_TEMP, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_PMIC_THERM},

{"625mv", CHANNEL_625MV, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"125v", CHANNEL_125V, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"pa_therm0", ADC_MPP_1_AMUX3, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_PA_THERM},

{"xo_therm", CHANNEL_MUXOFF, CHAN_PATH_SCALING1, AMUX_RSV0,

ADC_DECIMATION_TYPE2, ADC_SCALE_XOTHERM},

//<!--Add @2013-03-25 mingjun.liu

{"mpp6", ADC_MPP_1_AMUX5, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"mpp1", ADC_MPP_1_ATEST_8, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"mpp2", ADC_MPP_1_USB_SNS_DIV20, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"mpp3", ADC_MPP_1_DCIN_SNS_DIV20, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_DEFAULT},

{"mpp5", ADC_MPP_1_AMUX4, CHAN_PATH_SCALING1, AMUX_RSV1,

ADC_DECIMATION_TYPE2, ADC_SCALE_BATT_THERM_ECTH_3435K},

//--!>

};

- Gpio

<arch/arm/mach-msm/board-9615.c>

static struct gpio_keys_button msm9615_buttons[] = {

{

.code           = KEY_WPS_BUTTON,

.gpio           = 11,//MSM_GPIO_TO_INT(11),

.desc           = "wps_key",

.active_low     = 1,

.type		    = EV_KEY,

.wakeup		    = 1,

//.debounce_interval = 15,

},

//<!-- Add key @2012-01-08 mingjun.liu

{

.code		= KEY_CLEAR,//key code

.gpio		= 80,

.desc		= "setting reset",

.active_low	= 1,

.type		= EV_KEY,//类型

.debounce_interval = 15,

.wakeup		= 1,//Add @2012-01-10 mingjun.liu

},

{

.code		= KEY_SELECT,

.gpio		= 85,

.desc		= "sim switch",

.active_low	= 0,

.type		= EV_KEY,

.debounce_interval = 30,//Add @2012-01-10 mingjun.liu

.wakeup 	= 1, //Add @2012-01-10 mingjun.liu

},

//--!>

};

- Lc02-slic

原理说明：

<lc02/source/build/Makefile>

export SI3217X=$(TOPDIR)/apps/slic

…

si3217x_build:

@echo build si3217x

cd $(SI3217X) && make && make install

si3217x_clean:

@echo clean si3217x

cd $(SI3217X) && make clean

…

jffs2_build:mode_build dnsmasq_build  dhcp_build ppp_build udev_build httpd_build  si3217x_build start_stop_daem

on_build image_root www_build service_build

源码

<lc02/source/apps/slic/>

Kernel driver

<lc02/source/linux/kernels/mips-linux-2.6.31/drivers/telephony/si3217x.c>